# Ohm's law

Ohm's law states V = I x R: voltage equals current times resistance. Knowing any two values lets you calculate the third. MARKER-OHM-DOCX.

## Water analogy

Voltage is water pressure, the valve is the resistor, flow is current. Higher pressure or a more open valve raises the flow. MARKER-HOSE-DOCX.

## Triangle method

Cover V to get I x R; cover I to get V / R; cover R to get V / I. MARKER-TRIANGLE-DOCX.

## Board practical wiring

Connect battery positive to ammeter positive (red), ammeter negative to one end of the unknown resistor, the other end through the rheostat back to battery negative. Voltmeter goes in parallel across the resistor: red where current enters, black where it leaves. MARKER-WIRING-DOCX.

## Measured V-I pairs (R = 250 ohm)

| voltage_V | current_mA | resistance_ohm |
| --- | --- | --- |
| 3.0 | 12.0 | 250 |
| 6.0 | 24.0 | 250 |
| 9.0 | 36.0 | 250 |
| 12.0 | 48.0 | 250 |

Figure: the board circuit.